#include <Wire.h>

#include <Adafruit_TCS34725.h>

#include <Servo.h>

// =============== ПИНЫ ===============

#define PWMA 11   // Левый мотор

#define AIN1 6

#define AIN2 7

#define PWMB 3    // Правый мотор

#define BIN1 5

#define BIN2 4

// Ультразвук спереди — для цилиндров

#define TRIG_FRONT 12

#define ECHO_FRONT 13

// Ультразвук вниз — для пропастей

#define TRIG_DOWN 8

#define ECHO_DOWN 9

#define SERVO_PIN 10  // серво на пине 10

int motorSpeed = 50;

int escapeSpeed = 40;

// =============== ПОРОГИ ===============

const int VOID_DISTANCE = 9; // см

const int OBJECT_DIST = 6;   // см

// =============== ГЛОБАЛЬНЫЕ ===============

String carriedColor = "NONE";

const int TOTAL_OBJECTS = 8;

int objectsDelivered = 0;

bool isFinished = false;

Adafruit_TCS34725 tcs = Adafruit_TCS34725(TCS34725_INTEGRATIONTIME_154MS, TCS34725_GAIN_60X);

Servo myServo;

void setup() {

Serial.begin(9600);

pinMode(PWMA, OUTPUT); pinMode(PWMB, OUTPUT);

pinMode(AIN1, OUTPUT); pinMode(AIN2, OUTPUT);

pinMode(BIN1, OUTPUT); pinMode(BIN2, OUTPUT);

pinMode(TRIG_FRONT, OUTPUT);

pinMode(ECHO_FRONT, INPUT);

pinMode(TRIG_DOWN, OUTPUT);

pinMode(ECHO_DOWN, INPUT);

myServo.attach(SERVO_PIN);

myServo.write(15); // открыто

if (!tcs.begin()) while (1);

}

int getDistanceFront() {

digitalWrite(TRIG_FRONT, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_FRONT, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_FRONT, LOW);

long duration = pulseIn(ECHO_FRONT, HIGH, 25000);

return (duration == 0) ? 999 : duration * 0.034 / 2;

}

int getDistanceDown() {

digitalWrite(TRIG_DOWN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_DOWN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_DOWN, LOW);

long duration = pulseIn(ECHO_DOWN, HIGH, 30000);

return (duration == 0) ? 999 : duration * 0.034 / 2;

}

String detectColor() {

uint16_t r, g, b, c;

tcs.getRawData(&r, &g, &b, &c);

float total = r + g + b;

if (total == 0) return "WHITE";

float r_norm = r / total;

float g_norm = g / total;

float b_norm = b / total;

if (r_norm > 0.36) return "RED";

if (g_norm > 0.35) return "GREEN";

if (b_norm > 0.44) return "BLUE";

if (c < 12000) return "BLACK";

return "WHITE";

}

void forward(int spd) {

digitalWrite(AIN1, HIGH); digitalWrite(AIN2, LOW);

digitalWrite(BIN1, HIGH); digitalWrite(BIN2, LOW);

analogWrite(PWMA, spd);

analogWrite(PWMB, spd);

}

void backward(int spd) {

digitalWrite(AIN1, LOW); digitalWrite(AIN2, HIGH);

digitalWrite(BIN1, LOW); digitalWrite(BIN2, HIGH);

analogWrite(PWMA, spd);

analogWrite(PWMB, spd);

}

void turnRight() {

digitalWrite(AIN1, HIGH); digitalWrite(AIN2, LOW);

digitalWrite(BIN1, LOW); digitalWrite(BIN2, HIGH);

analogWrite(PWMA, escapeSpeed);

analogWrite(PWMB, escapeSpeed);

}

void stopMotors() {

digitalWrite(AIN1, HIGH); digitalWrite(AIN2, HIGH);

digitalWrite(BIN1, HIGH); digitalWrite(BIN2, HIGH);

analogWrite(PWMA, 0);

analogWrite(PWMB, 0);

}

void loop() {

// put your main code here, to run repeatedly:

int distFront = getDistanceFront();

int distDown = getDistanceDown();

String color = detectColor();

Serial.println(distFront);

Serial.println(distDown);

Serial.println(color);

// 🔴 АВАРИЯ: пропасть под роботом

if (distDown > VOID_DISTANCE) {

// Если держим груз — немедленно сбросить!

if (carriedColor == "NONE") {

stopMotors();

delay(200);

backward(motorSpeed / 2); delay(1000);

stopMotors(); delay(200);

turnRight(); delay(600);

stopMotors();

}

}

if (carriedColor != "NONE") {

forward(motorSpeed);

Serial.println(color);

// Любой люк — сбрасываем

if (color == "BLACK" || distDown > 9) {

delay(200);

stopMotors();

myServo.write(15); delay(1000);

backward(motorSpeed / 2); delay(500);

stopMotors();

carriedColor = "NONE";

objectsDelivered++;

if (objectsDelivered >= TOTAL_OBJECTS) isFinished = true;

}

return;

}

forward(motorSpeed);

// Избегаем люков без груза

if (color == "RED" || color == "GREEN" || color == "BLUE" || color == "BLACK") {